FINAL DRONE AND BASE STATION SPECIFICATIONS ISRO Robotics Challenge - URSC 2026 ¦ Active final configuration

| Team | IIC SIT | Team ID | 10144 |
| --- | --- | --- | --- |
| Institute | Sinhgad Institute of Technology, Lonavala | Document | Final active configuration - 11 July 2026 |

ASCEND is a GPS-denied autonomous quadcopter and automated docking station developed for indoor survey, target-image acquisition, coordinate logging, autonomous return, precision landing, wired data transfer, and battery charging. The specification below reflects the active field-round configuration.

1. Final Drone Specification

| Subsystem / Item | Final specification | Function / implementation |
| --- | --- | --- |
| Platform | Custom autonomous quadcopter | Indoor GPS-denied survey and docking platform. |
| Core airframe | Custom carbon-fiber frame; 10 mm OD x 8 mm ID T700 tubes, carbon-fiber plates and custom clamps | Rigid, lightweight structure with reduced vibration and clean payload mounting. |
| Core footprint | Approx. 328 x 254 mm chassis envelope | Compact central structure; propeller span excluded. |
| Structural PCB | Custom 2 mm two-layer power / lower structural PCB | Reduces wiring, integrates power routing and supports docking contacts. |
| Landing interface | Custom landing gear and mechanical docking interface | Supports controlled landing, active centering and pogo-pin mating. |
| Motors | 4 x T-Motor Navigator MN3110, 470 KV | Low-speed thrust control and stable lift for indoor autonomy. |
| Propellers | 14 x 4.7 in Pro-Range carbon-nylon, CW/CCW | Efficient lift with mechanically robust blades. |
| ESC | T-Motor F55A Pro III 4-in-1 ESC, 3-8S | Single integrated controller for all four brushless motors. |
| Battery | 4S lithium-ion, 2800 mAh | Main flight and embedded-electronics energy source. |
| Power sensing | APM/Pixhawk power module, XT60, 28 V / 90 A, 3 A BEC | Battery voltage/current telemetry and regulated flight-controller supply. |
| DC regulation | MINI560 5 V / 5 A buck + 200 W / 20 A adjustable buck | Regulated rails for Raspberry Pi, sensors and payload electronics. |
| Flight controller | Pixhawk 6C; STM32H743 Cortex-M7, 480 MHz, redundant IMUs | Attitude/rate control, motor mixing, estimator, arming and immediate failsafes. |
| Autopilot | PX4 | Position/altitude control, offboard interface and safety logic. |
| Companion computer | Raspberry Pi 5, 16 GB RAM, 512 GB NVMe SSD | Mission supervisor, logging, image handling and onboard AI processing. |
| Mission interface | Python mission software using MAVSDK / MAVLink | Pre-flight checks, takeoff, lane survey, RTL, landing and event logging. |
| Primary indoor sensing | SLAMTEC RPLIDAR A1M8, 360-degree 2D scan, approx. 6-12 m range | Arena-wall perception, lane support, obstacle awareness and safety boundary triggering. |
| Hover / altitude aid | Holybro H-Flow optical flow + integrated ToF rangefinder and IMU | Near-ground motion estimation, hover stabilization and altitude support. |
| AI camera | Raspberry Pi AI Camera, Sony IMX500, 12 MP with on-sensor neural processing | HD feature-image capture and visual inspection. |
| AI accelerator | Raspberry Pi AI HAT+ 2, Hailo accelerator, 40 TOPS | Accelerated inference and post-flight visual analysis. |
| Thermal sensing | Waveshare MLX90640, 32 x 24 pixels, 55-degree FOV, I2C | Auxiliary surface-temperature scanning and feature classification. |
| Communications | Pixhawk-RPi serial/USB MAVLink; Wi-Fi for monitoring; docked Ethernet for data offload | Separates flight-critical control from high-volume mission-data transfer. |
| Data pipeline | HD capture -> dock transfer -> LR conversion -> SIFT/KNN/RANSAC matching -> result logging | Target detection, coordinate record, proof-image storage and report generation. |
| Monitoring | Custom web dashboard and flight/mission logs | Mission state, active path, sensor health, detections and docking status. |
| Validated functions | Autonomous takeoff, survey, coordinate determination, return/landing support, charging and data validation | Demonstrated as the integrated challenge workflow. |

Configuration control: This sheet lists the active final stack. Earlier Livox MID360 / OAK-D development configurations are not part of this frozen specification unless physically reinstated before submission.

FINAL BASE STATION SPECIFICATION Automated docking, centering, charging, data transfer and post-flight analysis

| Team | IIC SIT | Team ID | 10144 |
| --- | --- | --- | --- |
| Institute | Sinhgad Institute of Technology, Lonavala | Document | Final active configuration - 11 July 2026 |

The v6.1 base station receives the landed drone, mechanically centers it, protects and actuates the docking interface, verifies electrical contact, transfers mission data through wired Ethernet, charges the 4S battery, and presents post-flight results on a local dashboard.

2. Final Base Station Specification

| Subsystem / Item | Final specification | Function / implementation |
| --- | --- | --- |
| Primary functions | Autonomous docking compatibility, minimum 5% battery increase, and LAN-to-LAN data transfer | Implements the challenge docking, charging and data-transfer tasks. |
| Landing platform | Approx. 22 x 22 in landing/docking area | Provides a defined return and mechanical-centering zone. |
| Main frame | 20 x 20 mm aluminum extrusion profiles | Rigid, modular and field-serviceable structure. |
| Joints / fasteners | Cast corner brackets; M5 x 0.8 x 25 screws, DIN 985 M5 lock nuts and washers | Resists loosening and maintains square alignment. |
| Landing surface | Smooth glass acrylic top with black acrylic side enclosure | Low-friction centering surface and protected internal hardware. |
| Active centering | Four-axis mechanical bed driven by 4 x NEMA 17 stepper motors | Moves the landed drone toward the docking center. |
| Lead-screw drive | T8 lead screws, 8 mm diameter, 1.25 mm pitch, custom brass nuts | Converts motor rotation into controlled linear motion. |
| Bearings | KP08 / KFL08 zinc-alloy pillow-block flange bearings | Supports screw rods and maintains concentricity. |
| Travel limits | 8 x Omron snap-action limit switches | Homing, end-stop protection and repeatable centering travel. |
| Dock connector | Custom 3D-printed spring-loaded pogo-pin plate | Compensates for small alignment and level errors. |
| Power contacts | Dedicated high-current positive, negative and BMS/charging contacts | Safe 4S battery charging through the dock. |
| Data contacts | Electrically isolated Ethernet pogo-pin group | High-speed wired mission-image, telemetry and status transfer. |
| Linear actuator | Vertically actuated pogo-pin mating mechanism | Engages contacts only after mechanical alignment is confirmed. |
| Iris shutter | Automated protective iris with linear-actuator interlock; 8 s close delay after actuator home | Protects connectors/electronics from dust, debris and rotor downwash. |
| High-level controller | Raspberry Pi 5, 16 GB RAM | Dashboard, file transfer, post-processing and station supervision. |
| AI processing | Raspberry Pi AI HAT+ 2 / Hailo-10H, 40 TOPS INT4, 8 GB accelerator memory | Local visual verification and target-analysis acceleration. |
| Real-time controller | Teensy 4.1 | Stepper pulses, limit-switch input, relay control and deterministic sequencing. |
| Motor drivers | 4 x TB6600 microstep drivers | Independent control of the four centering axes. |
| Display | 7 in IPS capacitive touchscreen, 1024 x 600, HDMI | Local mission status, images, detections and map visualization. |
| Charging architecture | 4S BMS-based balanced charging with heavy-duty relay; default relay state OFF | Power is enabled only after alignment/contact verification. |
| Charging evidence | Dock logs show battery-side rise from about 12.62 V to 14.08 V during testing | Confirms power transfer and charging progression through pogo pins. |
| Data processing | HD data reception, LR conversion, target matching, coordinate log and analyzed-target archive | Creates auditable post-flight results while preserving original HD evidence. |
| Control modes | FULL AUTO; SKIP ETH; SKIP CHG; MECH ONLY | Supports full operation and safe isolated field diagnostics. |
| Dashboard outputs | Original/annotated images, target labels, coordinate records, stitched arena map and system status | Centralized post-flight engineering HUD. |
| Safety provisions | Limit switches, safe-off relay, BMS, contact confirmation, actuator timeout/interlock and mechanical lock nuts | Prevents over-travel, accidental energization and unreliable docking. |

3. Automated Operating Sequence

| 1. LAND Drone returns and confirms touchdown | 2. CENTER Four axes align the drone | 3. OPEN Iris / actuator expose dock | 4. MATE Pogo pins establish contact | 5. LINK Ethernet connection is verified | 6. PROCESS Images and logs are transferred | 7. CHARGE Relay enables BMS charging | 8. RESET Actuator retracts and iris closes |
| --- | --- | --- | --- | --- | --- | --- | --- |